Homework 3

TP01: Calculate the sum of series of 8 bit numbers (no carry)

LXI H 2000

LDA 2010

MOV B A

MVI A 00

NEXT: ADD M

INX H

DCR B

JNZ NEXT

STA 2005

HLT

TP02: Calculate the sum of series of 8 bit numbers (with carry)

LXI H 2000

LDA 2010

MOV B A

MVI A 00

MVI C 00

NEXT: ADD M

JNC SKIP

INR C

SKIP: INX H

DCR B

JNZ NEXT

STA 2005

MOV A C

STA 2006

HLT

TP03: Calculate the sum of 1+2+..+n

LDA 2000

MOV B A

MVI A 00

MVI C 01

NEXT: ADD C

INR C

DCR B

JNZ NEXT

STA 2001

HLT

TP04: write an assembly language to calculate a square of a number

MVI H 07

MVI L 07

XCHG

MOV C D

MVI D 00

LXI H 0000

LOOP: DAD D

DCR C

JNZ LOOP

SHLD 2001

HLT

TP05: write an assembly language to calculate (a+b)^2 and store answer in location 2005

LDA 2000

MOV B A

LDA 2001

ADD B

MOV B A

MVI A 00

MOV C B

LOOP: ADD B

DCR C

JNZ LOOP

STA 2005

HLT

TP06: write an assembly language to calculate whether m X 2n and store answer in address 2000

MVI A 05

MVI C 03

LOOP: RLC

DCR C

JNZ LOOP

STA 2000

HLT

TP07: Write an assembly language to calculate (a-b)^2

MVI A 08

MOV B A

MVI A 03

MOV C A

MOV A B

SUB C

MOV D A

MOV E A

MVI A 00

LOOP: ADD D

DCR E

JNZ LOOP

STA 2000

HLT

TP08: Write an assembly language to calculate whether (a+b)^2 or (a-b)^2

MVI A, 01

MVI B, 08

MVI C, 03

CPI 01

JZ ADD

CPI 02

JZ SUB

ADD: MOV A, B

ADD C

MOV H, A

MOV L, A

MVI B, 00

MVI C, 00

JMP MULT

SUB: MOV A, B

SUB C

MOV H, A

MOV L, A

MVI B, 00

MVI C,00

JMP MULT

MULT: XCHG

MOV C, D

MVI D 00

LXI H 0000

LOOP: DAD D

DCR C

JNZ LOOP

SHLD 2000

HLT